Video Link: https://www.youtube.com/watch?v=r8trnHixvJs

WHAT WE NEED TO DO:

Cut the pre existing lines, cap them temporarily while we add piping length, reattach and add repressurize/add refrigerant

Summary:

- Identify the Refrigerant Type

  - Check the appliance nameplate (usually inside the fridge/freezer compartment or on the back panel)

  - Confirm it uses R-134a refrigerant before proceeding.

- Locate the Service Line

  - Find the low-side suction line (the larger diameter copper line coming from the compressor).

  - This is where the refrigerant will be added.

- Install a Bullet-Piercing (Saddle) Valve

  - Clamp the valve onto the suction line.

  - Tighten the piercing screw until it punctures the copper line, then back it out slightly to open flow.

  - This creates a temporary service port.

- Attach the Hose and Gauge

  - Connect the charging hose with pressure gauge to the valve.

  - Make sure connections are tight to prevent leaks.

- Connect Refrigerant Canister

  - Attach the can of R-134a refrigerant to the other end of the hose.

  - Some hoses have a quick-tap piercing mechanism to puncture and open the can.

- Purge Air from the Hose

  - Briefly loosen the hose at the valve connection to let a little refrigerant escape.

  - This removes trapped air from the line before charging.

- Start the Appliance

  - Turn the refrigerator/freezer ON so the compressor is running.

  - Refrigerant must be drawn in while the compressor is active.

- Introduce Refrigerant

  - Open the valve on the hose to allow refrigerant into the suction line.

  - Add refrigerant slowly, in short bursts, while monitoring the gauge.

  - Pressure should typically stabilize around 0–5 psi (suction side) when running for R-134a, but exact specs depend on model.

- Check Cooling Performance

  - Feel the evaporator coil/lines inside the fridge — they should get cold after a short time.

  - Monitor the freezer compartment temperature; it should start dropping within 15–30 minutes.

- Close and Seal the System

  - Once correct pressure and cooling are reached, close the piercing valve.

  - Disconnect the hose.

  - Leave the bullet-piercing valve in place (temporary but common for appliance service).

- Final Check

  - Listen and feel for any leaks around the valve.
Ensure the compressor is cycling normally and frost is forming evenly on coils.

Important Notes

- The video demonstrates a basic recharge method using a bullet-piercing valve and hose kit — useful for quick fixes, but not the professional “best practice.”

- A full service should include: evacuating the system with a vacuum pump, leak testing, and charging by weight with a scale.

- Overcharging can damage the compressor — it’s safer to add refrigerant gradually and check performance rather than dumping in too much.

- Always wear gloves and goggles when handling refrigerants.

List of Tools:

- Bullet-piercing (saddle) valve

- Gauge hose assembly, compatible with R-134a systems

- R-134a refrigerant

- Potentially can adapter (from automobile to fridge)

- Conversion valve or adapter for connecting hoses to the piercing valve

Possible Risks:

- Refrigerant leaking

- Overcharging the system and damaging the compressor