My thoughts:

TLDR: Cutting refrigerant piping leads to a whole lot of steps that have very little room for error. I am also not confident in our ability to even cut the refrigerant piping, the easier, smarter option is to simply bend the piping/coil assembly to create enough space for the ice casings

Honestly, cutting piping and adding flare fittings seems quite easy in practice. 
	Process to flare refrigerant lines

https://www.youtube.com/watch?v=itoro65YpZM

Tools required:

Pipe cutter

Flare fittings

However, for our application it is far more difficult. The place where we want to cut the piping is very tight, I don’t know if we can fit a pipe cutter, let along rotate it to actually cut the pipe

In addition to fitting the pipe cutter, once we cut any of the piping, the system will depressurize, and R145a refrigerant will leak out. From what I saw in the previous video here, at higher pressures R145a is a liquid, lower pressures it is a gas. It is apparently illegal to cut and reseal refrigerant piping without a professional license (link to epa article).

However, R145a is considered a slightly hazardous material (safety data sheet).

If we are able to cut the piping and assume the risks, we will still have to find some way to cap the cut end of the piping (the side of the compressor) to prevent refrigerant from leaking.

Oftentimes refrigeration cycles are designed to use a specific amount of refrigerant, so it is important to lose as little refrigerant as possible.

Assuming we can somehow cut the piping and reseal the ends, and we can fit flare fittings onto the news and are able to reattach everything, we still need a way to pressurize the whole system. There are many different options including adding dry nitrogen to the system to check for leaks.

Assuming there are no leaks, or we resolve any leaks, we must then bring the system to a vacuum, as well as check that the condenser and pump can pressurize.

Finally, we would then have to add the correct amount of refrigerant to restore the system. The best way to do this would be to bleed and empty the R145a out of the fridge, which is possible. Case in point the video we found earlier, I have no watched the whole thing

In my opinion, the two biggest obstacles I see are the space constraints, and repressurizing the system. From my memory, I do not know if we can fit a pipe cutter on the pipe (link to pipecutter)

The second biggest obstacle would be leak checking. From google and youtube, everybody recommends DIYers to ignore any refrigerant piping leak, it is not worth searching for, and is far easier to just top off the system every few years with more refrigerant.

By not cutting the lines, we avoid all of this, and we don’t even have to add refrigerant. It will be tricky to bend the existing piping and coil exchanger to create the 6 inches of space for the casing assembly. But as long as we go slow, maybe use a heat gun, there are far less complications involved (we just can’t break the piping)

Below I have attached me asking Chatgpt a few questions to give me appoint of reference to research into:

can you outline the steps to safely cut a refrigerator's coolant lines and reattatch them?

###### ChatGPT said:

Got it — you’re asking about cutting and reattaching a refrigerator’s refrigerant lines. I need to flag something important up front:

⚠️ Caution: Refrigerators use refrigerants (R134a, R600a, R290, etc.) that are either ozone-depleting, flammable, or both. U.S. EPA Section 608 requires certification to service, recover, or recharge systems containing refrigerants. Venting refrigerant is illegal and unsafe. If you’re not certified, the safe/legal path is to have a licensed HVAC/R technician handle it.

That said, if your goal is just to understand the process, here’s a safe outline of the professional steps (not a DIY guide):

### 1. Preparation & Safety

- Disconnect power to the refrigerator.

- Identify refrigerant type from the data plate (R134a, R600a, etc.).

- Work in a ventilated area — especially critical for flammable refrigerants like isobutane (R600a).

- Gather tools: recovery machine & tank, tubing cutter, brazing torch, nitrogen, vacuum pump, manifold gauge set, filter-drier, PPE (gloves, goggles).

### 2. Recover Refrigerant

- Connect a recovery machine and certified recovery tank.

- Use service valves or pierce valves (if no access port exists).

- Recover refrigerant fully until system is at 0 psig.

### 3. Cut the Lines

- Use a tubing cutter (never a saw, to avoid metal shavings).

- Cut the suction and liquid lines at clean, accessible points.

- If replacing a component (compressor, evaporator, etc.), cut close to the failed part.

### 4. Reattach / Reconnect Lines

- Clean and deburr tubing ends.

- Insert a new filter-drier (always replaced when the sealed system is opened).

- Braze connections using silver-bearing brazing rod (or soft solder for very small R600a systems, depending on manufacturer guidance).

- Purge with dry nitrogen during brazing to prevent oxidation inside the tubing.

### 5. Pressure Test

- Pressurize system with dry nitrogen (e.g., 150 psig) and check for leaks using soap solution or electronic leak detector.

### 6. Evacuation

- Connect vacuum pump and pull system down to at least 500 microns.

- Isolate pump and verify system holds vacuum (no leaks/moisture).

### 7. Recharge System

- Weigh in the exact factory-specified refrigerant charge (critical for performance).

- Use a scale — small appliances are very charge-sensitive (±2 grams can matter).

### 8. Restart & Verify

- Power on refrigerator.

- Check pressures, frost pattern, and amp draw.

- Monitor for proper cooling.

✅ In short: the key steps are recover → cut → braze → pressure test → vacuum → recharge → verify.

Do you want me to adapt this into a DIY-friendly workaround (like safe alternatives for line splicing or sealing without refrigerant handling), or would you prefer I stick to the professional EPA-compliant process?

##### You said:

a DIY friendly approach would be awesome

###### ChatGPT said:

Perfect, thanks for clarifying. I’ll walk you through a DIY-friendly approach — but first, two important notes:

- Legality & safety: Venting refrigerant into the air is illegal and can be hazardous. That means a true cut and reattach job usually isn’t DIY-safe unless the system is already empty (for example, if it leaked down on its own, or you’re dealing with a practice unit).

- Tiny tolerance: Refrigerators are very charge-sensitive — a few grams off can ruin performance. So even if you splice lines successfully, recharging without the right refrigerant type and scale is the real bottleneck.

That said, here are a couple of DIY-adapted options depending on what you’re trying to do:

## 🔧 If you need to physically cut and rejoin lines (for routing or repair)

- Unplug the fridge and make sure the system is fully depressurized (if there’s still refrigerant in it, stop here).

- Cut the line with a small tubing cutter (not a hacksaw).

- Rejoin options without brazing:

  - Compression fittings (flare or ferrule type) — clamp over the tubing, tighten to seal. Easier than brazing, but more prone to leaks.

  - Push-to-connect fittings made for refrigeration tubing (like Lokring or equivalent). These crimp around the tube with a tool and create a permanent seal.

  - Swaging & soldering (if you’re comfortable with a torch, soft solder and flux can work on small R600a systems — less demanding than brazing).

- Seal well: Make sure the joint is snug and airtight, otherwise you’ll lose vacuum/refrigerant when the system is charged.

## 🔧 If you don’t plan to recharge (DIY bypass)

If the unit has already lost refrigerant and recharging isn’t practical:

- You can seal off cut lines with crimping + solder or epoxy to stop oil leakage and odors.

- The fridge won’t run as a fridge anymore, but it can be repurposed as an insulated cabinet or kegerator shell.

## 🔧 Safer DIY alternatives instead of cutting/rejoining

Sometimes folks cut into coolant lines because they want to:

- Move the evaporator plate

- Add a viewing window

- Convert a mini-fridge for fermentation, etc.

In these cases, safer options are:

- Re-route wiring or shelving instead of cutting refrigerant lines.

- Work around the evaporator plate — bending gently if needed, without breaking the sealed tubing.

- Use an external temp controller (like Inkbird) instead of altering the refrigeration system.

👉 Bottom line: A full “cut → reconnect → recharge” cycle really needs pro tools (vacuum pump, nitrogen, refrigerant, scale). But for DIY, you can:

- Use mechanical fittings (compression, Lokring) if you must reconnect tubing.

- Seal off ends if you just need to cut and cap.

- Avoid refrigerant handling altogether by working around the sealed system where possible.

Would you like me to draw up a step-by-step “minimal-tool DIY method” (using just a cutter, compression fitting, and epoxy seal), so you’d have the simplest safe version?

##### You said:

yes please

###### ChatGPT said:

Great 👍 — here’s a minimal-tool DIY method for cutting and reattaching refrigerator coolant lines without brazing or specialized HVAC tools. This assumes the system is already empty of refrigerant (⚠️ if it isn’t, stop — venting is unsafe/illegal).

# 🛠️ Minimal-Tool DIY Method

### Tools & Materials

- Small tubing cutter (cheap handheld type for 1/4" – 3/8" copper/aluminum lines)

- Compression fittings (or push-fit refrigeration fittings like Lokring if you can find them)

- Adjustable wrench

- Sandpaper/emery cloth (to clean tubing ends)

- Optional: two-part epoxy or JB Weld for sealing reinforcement

### Step 1: Power & Safety

- Unplug the refrigerator.

- Work in a well-ventilated space.

- Wear gloves and safety glasses — the tubing can have sharp edges.

### Step 2: Cut the Lines

- Use the tubing cutter to cut the refrigerant line cleanly.

  - Tighten the cutter slowly, rotating until it scores and slices through.

  - Avoid squashing the tube.

### Step 3: Prep the Tubing Ends

- Use sandpaper/emery cloth to lightly clean the outside of the cut ends.

- Make sure the cut is round and burr-free so the fitting seals well.

### Step 4: Attach Compression Fitting

- Slide the compression nut and ferrule (ring) onto one tube end.

- Insert the tube into the fitting body until it stops.

- Slide ferrule and nut into place, then tighten the nut with your fingers.

- Use the wrench to snug it down further (about 1 full turn past hand-tight).

  - Don’t overtighten — it can crack small tubing.

### Step 5: Seal & Reinforce

- Once the fitting is tight, you can add a small amount of epoxy or JB Weld around the outside for extra leak resistance.

- Let it cure fully before moving or testing.

### Step 6: Secure the Line

- Use zip ties or clamps to prevent vibration at the joint (compression fittings can loosen if stressed).

✅ Result: You’ll have a mechanically sealed splice without brazing.

⚠️ But note: This won’t restore refrigerant — the fridge will only cool again if the system is evacuated and recharged by a pro. If you just need a sealed, reattached line (to stop oil leaks, prevent smell, or keep tubing intact), this method works fine as a DIY fix.

Would you like me to also give you a DIY “cut and cap” version (for when you don’t need to reconnect the line at all, just safely seal it off)? That’s sometimes even easier.