| Reducing Energy Costs Using Thermal Mass in Refrigerator |  |
| --- | --- |
| Student Team Members: Isabel Price (Lead) Matthew Manuel Brent Marin Caleb Mok Kalino Ruiz Faculty Advisors: Dr. Keith Yi Sargon Ishaya |  |
| Project Objectives: We implemented a passive cycle assembly to a refrigerator that blows air over a thermal mass to maintain a safe internal temperature during the peak utility rates, reducing electricity cost, and a microcontroller to control the active and passive modes. - Utilize thermal storage to shift load away from peak time of use duration while maintaining safe refrigeration temperatures - Evaluate performance through energy consumption of both active and passive cooling cycles - Calculate money saved through minimizing energy usage during peak hours - Evaluate the air velocity profile of the passive cooling cycle Significance: This smart refrigerator improves environmental sustainability through reducing electricity load and carbon emissions during peak utility rate periods. Project Results: - The fan successfully cycles on when the temperature is below 40°F until the chamber temperature lowers to 37°F. - The expected energy savings are 24.0 kWh/year based on initial tests. - The resulting cost savings are $12.60 per year. - The velocity profile was determined using an anemometer with an average velocity of 1.14 m/s. Sponsors: Thank you to Dr. Keith Yi, Sargon Ishaya, Desmond Cheung, Christopher Price, and Dr. Hussameddine Kabbani for their support throughout the design and fabrication of our senior design project. |  |